# Networking

## Making VLAN

### In a switch

First create the vlans, all vlans have to be make in all the switches

Then you need to configure the native Vlan and if necessary the default gateway for the switch

Then you need to assign the interfaces of the switch where they have the host connected to the right Vlan and configure It in access mode

if there is more than 1 interface use the range command

You need to configure the connection between the switches and router like in trunk mode and setting which vlan traffic is allow on each trunk

assign a default gateway for each regular switch, so they can be manage remotely

### In the Router

we need to identify each Vlan in the interface that’s coming from with the interface.vlan, and enable the encapsulation also the main interface if you want we can give a description and activate no shutdown(IN A LV3 SWITCH IS NOT NECESSARY TO PUT THE .VLAN OR ENCAPSULATION, WE JUST ACTIVATE THE VLANS IN THE SWITCH AND CONFIG THE IP AND MASK AND ACTIVATE IP ROUTING IN THE SWITCH)

### Setting the ip(default gateway) for each Vlan

in the router or lv3 switch

(config)#interface vlan  __

(config-if)# ip address ____ and the mac ___

add description if necessary

### With lv 3 switch and router

Set the interface that connects the lv3 switch and the router as a no switchport (it will become router port) and out a ip address and mask so it will serve as a default gateway for the vlans

Then remember to activate the ip routing option on the lv3 switch

Now configure the routing protocol OSPF for announcing each Vlan on the switch, the network ip has to be the ones from each pc and the network that the switch and router share.

### Router/switch commands to check vlans

- show ip route

- show ip interface brief – router trouble

- show interfaces – router trouble

- show interfaces trunk

- show vlan brief – missing vlans

- show interfaces switchport -missing vlans

- show interfaces trunk – trunk port problems

- show running-config – trunk port problems

## Spanning tree

When is portfast it goes from locked to online in a very few seconds avoiding other middle states

nfvis(config-if)# spanning-tree portfast enable or default ( set default) global or single interface (global gives issues sometimes)

nfvis(config-if)# commit

nfvis(config-if)# end

### Change spanning-tree port cost

nfvis(config)# interface range / or single vlan 10,30….

nfvis(config-if)# spanning-tree vlan 1,20,10 cost(change port cost) / port priority (same price choose one)

### Commands

nfvis(config)# spanning-tree vlan __ priority 12288 - do this in every switch

nfvis(config)# commit

nfvis(config)# end

nfvis(config)# spanning-tree enable

nfvis(config-)# commit

nfvis(config-)# end

nfvis(config)# interface gigabitEthernet 1/1

nfvis(config)# spanning-tree vlan __ root – defines if its primary or secondary bridge port

nfvis(config)#spanning-tree vlan _ , _ , _ , _ , root primary / secondary or priority → this sets the switch as the root bridge

making switch root bridge for vlans

spanning-tree vlan _ , _ , _ , _ , root primary / secondary or priority → this sets the switch as the root bridge

## EtherChannel

### Protocols

### PAgP- states (cisco only)

- on

- desirable

- automatic

### LACP states

- On

- Active

- Passive

### Requirements

- All interfaces must allow etherchannel

- Speed and duplex should be the same

- Vlan consistency must be the same in the interfaces

- The allow range for trunking should be the same on the etherchannel

### Config

Choose the range interfaces and assign them a group number and a protocol mode, then config the interface as trunk and define which vlans are allow to run trough

Also config the native

### Commands to check etherchannel

- show interfaces port-channel 1

- show etherchannel summary

- show etherchannel port-channel

- show interfaces f0/1 etherchannel

## DHCPV4

### Steps

CLIENT SEND CLIENT BROADCAST DHCPREQUEST 🡪SERVER ANSWERS WITH DHCPOFFER SAYING IM A SERVER HERE IS YOUR ADDRESS

CLIENT ACCEPTS ADDRESS WITH DHCPREQUEST WITH A BROADCAST 🡪SERVER SENDS DHCPACK LIKE A VOUCHER IN UNICAST

### CONFIGURE IN ROUTER ONLY OR LV3 SWITCH

First we need to add the excluded address for each vlan or lan that we don’t want dhcp to give automatically

Router(config)# ip dhcp excluded-address low-address [high-address]

Then we need to assign a pool name for all the ip pool for each vlan or lan

Router(config)# ip dhcp pool pool-name
Router(dhcp-config)#

After both are done and we are in the dhcp-config we can start adding the network id for each lan or vlan we are using and the mask, then the default router ip of the vlan or lan and the dns server ip, also the domain name (Optional)

To enable or disable dhcp we can use this command

### To configure router or switch as dhcp client

Instead of adding ip address write dhcp and it will automatically ask for an ip

### Commands to check DHCP

show ip dhcp binding – it show all ipv4 assign by the server

show ip dhcp server statistics

show running-config | section dhcp

in pc

ipconfig /renew /release

## SSH

switch

config# ip domain name choose name

config#user name admin secret cisco

config# crypto key generate rsa

bits 1024

config#line vty 0 4

config-line# transport input ssh

config-line# login